# SCATalogue User’s Guide

SCATalogue, version 1.61

Table of Contents

Introduction	3

What is SCATalogue?	3

NOAA Shoreline Oiling Summary form (SOS).	5

SCATalogue Forms	7

Initial Data Entry	7

Forms and Subforms	8

Survey List Form	8

Switchboard Form	9

Survey Form	9

Team Subform	11

Segment Form	13

Photos	14

Shoreline Treatment Recommendations Subform	15

Surface Oiling Form	16

Zones	17

Shoreline Treatment Recommendation (STR) & Notes	17

Photos	17

Subsurface Oiling Form	18

Shoreline Treatment Recommendation (STR) & Notes	18

Photos	19

Map Form	19

Sketch Form	20

Photo Form and Subform	21

Photo Metadata Subform	22

Data Accuracy & Completeness Subform	23

SCAT Data Transfer	24

Data Collection Protocols	24

Troubleshooting	24

Data Transfer	24

Appendix	25

GPS	25

Photos	25

Customization	26

Form Operations	26

# Introduction

The SCATalogue app is a creation of the California Department of Fish and Wildlife, Office of Spill Prevention and Response (CDFW-OSPR).

SCATalogue simplifies the collection of shoreline cleanup assessment technique (SCAT) data during a spill response. Survey data attributes are based upon the National Oceanic and Atmospheric Administration (NOAA) standard SCAT shoreline oiling summary (SOS) paper form. A SCAT survey is a collection of SCAT data for a length of shoreline called a given shoreline segment. SCATalogue is designed to support spill response SCAT data collection in the marine environment.

The survey data can be transferred via email, via iTunes or other methods if available.  These other methods are dependent on the user and/or user's organizational iPad setup and/or information technology (IT) protocols.

SCATalogue is designed for use on an iPad, specifically the iPad mini that is used by the California Department of Fish and Wildlife, Office of Spill Prevention and Response's field response staff.

CDFW OSPR's geographic information systems (GIS) unit utilizes ESRI ® GIS software almost exclusively.

Data output is in JSON format. Data containing geographic/spatial data is formatted in Environmental Systems Research Institute (ESRI®) geoJSON format. This output can be converted into to feature classes using ArcGIS Desktop's© ArcToolbox's "JSON To Feature" tool. CDFW OSPR GIS processes the JSON/geoJSON output from SCATalogue using this tool and other custom scripts/tools in ArcGIS Desktop© software.

# What is SCATalogue?

SCATalogue is an iOS app, built using Apple’s Xcode® integrated development environment (IDE). The software language of development is Swift 3.

It is an electronic field data collection app designed to collect shoreline cleanup assessment technique (SCAT) data.

The SCAT method originated during the response to the 1989 Exxon Valdez oil spill.  Less than a year later on February 7, 1990, the American Trader spilled approximately 416,598 gallons of crude oil off Huntington Beach in Southern California. These events inspired the California Legislature to enact legislation in 1990 called the Lempert-Keene-Seastrand Oil Spill Prevention and Response Act.

The Act covered all aspects of marine oil spill prevention and response in California. It established an Administrator who is given very broad powers to implement the provisions of the Act. The Act also gave the State Lands Commission certain authority over marine terminals. In 1991, the Office of Spill Prevention and Response (OSPR) launched, headed by the Administrator.

SCATalogue’s interface is form driven. Data is entered on a series of related forms.  Entry via forms was chosen versus a mapping interface because the basis for all SCAT collection has been and continues to be the National Oceanic and Atmospheric Administration (NOAA) SCAT paper form. It is intended to reflect the familiar input process for the SCAT teams.

## NOAA Shoreline Oiling Summary form (SOS).

There are eight sections on the SOS form.

SCATalogue was created by California Department of Fish and Wildlife - Office of Spill Prevention and Response (CDFW-OSPR). Because of its origins at CDFW-OSPR, some features of SCATalogue are more specific to a spill response in California.

The app contains picklists to simplify, reduce error and speed data entry. Some of the picklists default items contains list items applicable to California and some elements specific to CDFW-OSPR. For example, SCATalogue is an iOS app and CDFW-OSPR did not create an Android version due to the hardware platform limitations imposed by our organization. Further, it was optimized for the iPad Mini, although it will run on other iPads.

# SCATalogue Forms

SCATalogue has ten main forms and a number of subforms. The main forms roughly correspond to the eight sections on the NOAA SOS form.

The flow generally follows the SOS paper form from top to bottom going from section 1 to section 5. Once these data have been recorded, sections 6 (surface oiling) and 7 (subsurface oiling) can be accessed.

Many of the main forms have buttons to access subforms. These subforms allow the user to add secondary data. For example, for each surface oiling zone, a user can associate shoreline treatment recommendations (STR) via the STR subform.

The user can also associate up to two photos for each surface oiling zone. This is done via the Photo subform. Note that the Photo subform can also be accessed from the Subsurface Oiling form, the Segment form, and the Survey Switchboard form.

If a form has a navigation button on the top left, this goes back to previous form.  Repeated back navigation will take the user to the Survey Switchboard, then to the Survey List form.

Items/words in blue, are buttons. If the button is greyed out then it has been disabled.

# Initial Data Entry

SCATalogue directs a user during the initial data entry for a new survey. It does so by restricting access to just the form(s) and button(s) the user needs in order to initialize a survey. Afterwards, all forms are available. Not all data fields are required to initialize a survey but there is a minimum. See the appendix for more information.

The first form the user will see is the Survey List form.

# Forms and Subforms

## Survey List Form

The Survey List form allows users to manage their SCAT surveys. Initially, this list will be empty.

Each item in the Survey List is a unique survey. Each one represents a digital SOS form, essential. The “Status” column indicates the survey’s status. A survey can be either “active”, “completed”, or “exported”.

There can be many “exported” and “completed” surveys, but only one survey can be “active”.

When the user taps an individual survey on the list, the survey’s status determines which of the five buttons are enabled or disabled. Items or words in blue are buttons. If the button is greyed out then it has been disabled.

## Switchboard Form

The Switchboard form allows the user to navigate to any of the main forms. The status of the selected survey will determine which of these buttons are enabled or disabled.  The navigation button on the top right of the form allows the user to navigate back.

## Survey Form

The Survey Form corresponds to sections 1 and 2 of the NOAA SOS form.

The data entry fields in SCATalogue utilize picklists, checkboxes, and other mechanisms to make data entry easier and less prone to error. The user tapping the “Survey By” field, which provides a picklist, is shown below.

### Team Subform

The Team Subform is accessed by tapping the “Team” button on the Survey Form.  This subform manages the team numbers and their constituent team members.

Team members are added to the teams in one of three ways.

- “Download Team Assignments” is currently unavailable, but this will download the SCAT coordinator’s preset team assignments from an organizations’s cloud.

- “Lookup Contact” allows the user to add a member to a team from their iPad’s contact list.

- “Ad Hoc Entry” allows the user to add a member to a team using free text entry.

Note: Using “Lookup Contact” initially populates into the “Ad Hoc Entry” fields, allowing the user to edit as necessary and to assign the member to a specific team number. Tapping the team member in the lookup table also will add them to the “Ad Hoc Entry” fields, allowing the entry to be edited.

## Segment Form

The Segment Form corresponds to sections three, four, five, and eight of the NOAA SOS form.

Note: the “End GPS” button should only be tapped at the end of the segment and all data has been collected. Select this last.

Note: to indicate presence of the secondary ESI type; tap once to check. To indicate that this ESI type is oiled, tap again highlight. To uncheck, tap a third time. See figure# 3A, 3B, 3C respectively.

Note: there is a camera icon/button on the bottom of the form. The numbers zero and four on either side of the camera icon indicates how many of the four photos available have been taken. The camera icon and numbering appear on any form that allows photos.  The maximum number of available photos varies depending on the form.

Section 8 data is shown on the SCATalogue Segment form as “STR & Notes”. Tapping “STR & Notes” will open the Shoreline Treatment Recommendations subform.

After entering data into the STR subform and returning the Segment form, the data will appear in textboxes below the “STR & Notes” button. (See “Shoreline Treatment Recommendations Subform” section below.)

### Photos

The user can record up to four photos per segment.  Tap the camera icon at the bottom center of the Segment form. (See “Photo Form/Subform” section below.)

## Shoreline Treatment Recommendations Subform

Tapping on the “Shoreline Treatment Recommendations” button will activate a picklist.  Each selection from the picklist will be added to the table. The Shoreline Treatment Recommendations (STR) subform can be called by either the Segment, Surface Oiling, and Subsurface Oiling forms. The Notes textbox is free text and allows 1024 characters.  The STR table allows for as many entries as needed.  In the screenshot above, the navigation button on the top left indicates this instance of the STR subform was opened from the Surface Oiling form.

## Surface Oiling Form

The Surface Oiling Form corresponds to section 6 of the NOAA SOS form.

### Zones

On SCATalogue’s Surface Oiling Form below the “Save Zone” button is the zone table. This table’s rows represent the same data as the rows on the NOAA SOS form section 6. Each zone record begins with its Zone ID.

On the Surface Oiling form, the area above the “Save Zone” button is the currently selected zone’s “editing workspace”.

To start recording a new oiling zone, tap “Start GPS”. Then fill in, Zone ID, ESI Type, Tidal Zone, and Width (required fields). Then “Save Zone”. This will add a new zone record to the zone table at the bottom of the Surface Oiling form. Walk this zone assess and record the conditions. At the end of the zone tap, “End GPS”.

To edit or update an existing oiling zone, tap its zone table record. The data for this selected zone will populate into the “editing workspace.” After making any necessary edits/updates, tap “Save Zone”.

No Oiling Observed Zones (NOO) may be either explicit or implicit. Explicit: NOO zone data is collected as if any other type of oiling zone data. Implicit: SCAT coordinator must agree and communicate with SCAT teams that the survey without any positive oiling zone data will be considered and processed as a NOO zone.

CDFW-OSPR explicitly collects NOO zones just like any other oiled zone.

### Shoreline Treatment Recommendation (STR) & Notes

A Shoreline Treatment Recommendation can be entered for each zone. Tap the zone table record, then tap “STR & Notes” at the bottom right of the Surface Oiling form. Fill out the STR & Notes, then navigate back to the Surface Oiling Form. (See the “Shoreline Treatment Recommendations Subform” section above.)

### Photos

Two photos can be recorded for each zone. Tap its zone table record, then tap the camera icon at the bottom center of the Surface Oiling form. (See the “Photo Form/Subform” section below.)

## Subsurface Oiling Form

### Shoreline Treatment Recommendation (STR) & Notes

A Shoreline Treatment Recommendation can be entered for each pit/ trench.  Tap the zone table record, then tap “STR & Notes” at the bottom right of the Surface Oiling form. Fill out the STR & Notes, then navigate back to the Subsurface Oiling Form.  (See the “Shoreline Treatment Recommendations Subform” section above.)

### Photos

Two photos can be recorded per pit/trench. Tap its pit/trench table record, then tap camera icon at the bottom center of the Subsurface Oiling form. (See the “Photo Form/Subform” section below.)

## Map Form

The map form can orient the user start of a new survey to help the user locate the segment(s) in the vicinity. Since, SCATalogue is designed to be used offline; it has some mapping data built in. The layers include: California counties, county seats, selected cities/towns, major highways, major rail, major waterways, coastal points of intererest, and SCAT segments. Map layers are scale dependent.  With Wifi access, the map form will also have ESRI streets and satellite image as background options.

Note: the photo form has a “Send to Sketch” button which will copy the current displayed map and send that to the Sketch form to be used as a background for the sketch.

To do this, you go to the map form; zoom in on the area you are going to survey and tap the "Send to Sketch" button.

A new sketch will be created with the map screen capture as the background.

This way, when you subsequently sketch; it'll be on top of that map screen capture so you'll have references to help with your sketching and scale.

NOTE: You can "send to sketch" once a survey has been initiated, during a survey, after the survey segment has been completed, BUT not after it has been exported (sent to GIS).

## Sketch Form

The sketch form supports four sketches per survey.  Team members may also sketch on paper then take a photo of the paper sketch using SCATalogue’s Photo form. This also applies to any survey notes, etc. written on paper.

When utilizing, “Send to Sketch”; The background map can be the default, or if you have hot spotted your iPad to your phone (or have WiFi); then you can also use either the streets background or satellite imagery background.

See examples below..

## Photo Form and Subform

The Photos form can be opened from a number of the other forms. These photos then become associated with that data type (zone, pit, segment, etc). If opening the photo form directly, the data type is an “incidental photo”.

Note: Any photo can be annotated.

If an incidental photo, record in the metadata if it is associated with oiled wildlife.

### Photo Metadata Subform

## Data Accuracy & Completeness Subform

# SCAT Data Transfer

Data may be transferred from SCATalogue or iPad via email, flash drive, iTunes, local WiFi, or other means depending on the information technology setup.

# Data Collection Protocols

To ensure completeness, if the entire length of the assigned segment has been surveyed, check the “Entire Segment Surveyed” checkbox on the Segment Form next to the “NOO” checkbox.

Before ending the survey, go to the map and verify that the oiling zones, pit/trenches, etc. are depicted correctly. If there are discrepancies, correct them if possible. If they cannot be rectified, include instructions on how to do so to OSPR GIS within the transfer email or separate email.

No Oiling Observed Zones (NOO) may be either explicit or implicit. Explicit: NOO zone data is collected as if any other type of oiling zone data. Implicit: SCAT coordinator must agree and communicate with SCAT teams that the survey without any positive oiling zone data will be considered and processed as a NOO zone.

CDFW-OSPR does NOT explicitly collect NOO zones.  However, you can if desired.

# Troubleshooting

## Data Transfer

Generally, SCATalogue packages up the SCAT data into a .7z/zip file format.  It then, hands it off to another app to do the actual transfer of the data.  Which other app depends on the user’s choice such as email, AirTransfer, OneDrive, etc…

The most frequently used method has been via email.  The most common issue with email transfer is an old user password.  In order to avoid this issue, on your iPad; go to “Settings” -> “Passwords & Accounts” -> “O365”.  At this point, if your password is old; you may be prompted to enter in your current password.  If not, tap on your Email address and a dialog should appear where you can set your password. This is your CDFW network/Outlook password.

# Appendix

## GPS

SCATalogue uses the onboard GPS of the iPad. However, it can also utilize any Bluetooth GPS as long as it is compatible with the iPad.

## Photos

Image Naming Convention

COUNTY_DIVISION_SEGMENT_YYYYMMDD_HHMMSS_type<_ID><_PHOTONUM>.ext

type = incidental, segment, surfoil, suboil, signature, sketch

<_ID> = ZONEID (A1, A3), PITID# (1,4)]

<_PHOTONUM> = (1, 2, 3), NOT needed with signature

ext = jpg, png (jpg = photo, png = annotation)

HM_G_S034_20170718_140355_incidental_1.jpg

HM_G_S034_20170718_140355_incidental_1.png

LA_H_S005_20170525_143438_segment_3.jpg

LA_H_S005_20170525_143438_segment_3.png

HM_G_S034_20170718_140355_surfoil_A2_1.jpg

HM_G_S034_20170718_140355_surfoil_A2_1.png

OR_A_S005_20170606_132554_suboil_1_1.jpg

OR_A_S005_20170606_132554_suboil_1_1.png

AL-B-S000_20170511_102514_signature.jpg

AL-B-S000_20170511_102514_sketch_1.jpg

AL-B-S000_20170511_102514_sketch_1.png

## Customization

See SCATalogue Customization Guide document.

## Form Operations

Items and /or words in blue are buttons. If the button is greyed out then it has been disabled.

Some of the buttons show job aids. For example: “Show NOAA Job Aid” will show the NOAA’s Shoreline Assessment Job Aid PDF. On the Surface Oiling form, the “Distribution (%)” button will show the Percent Coverage Chart below.

On all forms that contain a primary and secondary data type such as Primary Backshore Character and Secondary Character as on the above Segment form, the primary is presented as a picklist and the secondary as checkboxes. Tapping the Secondary checkbox the primary, the secondary value will automatically populate into the primary.

#### Minimum Data Requirements

When creating a new survey-segment The Survey form and Segment form require data in certain information fields.

These fields are from the NOAA SOS sections:

- General Information

- Survey Team

- Segment

- a) Backshore Character

b) ESI Shoreline Type

- Operational Features

There are many fields to be filled out in these sections.

However, in SCATalogue the fields are mostly pick lists or checkboxes and/or semi or fully automated, there is little free text entry.

And, of that; there only EIGHT items that are actually required.

The others can be deferred until later as the segment will need to be accessed before recording the other data.

The Survey form minimums (five items) are listed below, followed by an example value.

Spill ID: 2017-SPILL-6

Segment ID: ALA-A-S001

Date: 01/30/2017 (remember, “Date:” is blue so is a button. Tap that to automatically fill in today’s date and time)

Start Time: 13:05:16

Team: 2

TAP “Next>”

On the bottom right of form.

The Segment Form minimums (three items) are listed below, followed by an example value.

Start GPS: 38.57654   -121.48397

Primary Backshore Character: Beach

Primary Shoreline ESI Type: 3A

TAP “Save”

On the bottom right of form.

NAVIGATION BACK USING “< Survey” before tapping “Save>” will cause the data to be lost.

“Save>” saves the entered data and guides the user back to the Switchboard Form.

In this view all the forms are accessible.

Survey and Segment with the required minimum data, Surface Oiling, Subsurface Oiling, Photo and Sketch

For further information about SCATalogue, please refer to the SCATalogue website or contact CDFW OSPR GIS at osprgis@wildlife.ca.gov

Information about Shoreline Cleanup and Assessment Technique (SCAT) can be found on the NOAA website.